Department of Electrical and Computer Engineering & Computer Science

Tagliatela college of engineering

CSYS 4483-01/CSYS 6683-01

Network Defense

Fall 2025

Instructor: Dr. Tirthankar Ghosh

## Lab 1: Packet crafting with Scapy

# Submission deadline: September 9. Upload the report on Lab 1 assignment folder under Assignments on Canvas. One report per group.

## Lab Overview

TCP has some serious security flaws in its design. RFC 793 does not prevent payload being transported on a TCP connection request packet (with SYN flag set). However, this is not the normal operation of TCP. If payload is transported on a connection request packet, it is left to the implementation of the TCP/IP stack to craft a response. Nonetheless, the vulnerability can be exploited to port malicious code to a server. This exercise is designed to provide hands-on exposure to craft TCP SYN segments with payload, and study response.

## Objective

The main objective of the exercise is to craft TCP SYN segments with payload, and study response.

Systems needed:

- Ubuntu/Fedora Linux VM (target)

- Kali Linux VM (attacker)

## Implementation Steps

Carry out the following steps on the Ubuntu/Fedora Linux VM

- Install apache web server

- Create a test page (index.html under /var/www/)

- Edit the httpd.conf or apache.conf file to listen on the interface IP address and port 80

- Start apache2

- Stop any firewall service that maybe be running (iptables, firewalld)

- Install Wireshark

- Launch Wireshark

- On the Wireshark window, go to Capture, Interfaces, and choose appropriate interface to capture

- Start capture

Carry out the following steps on the Kali VM

- Open an editor and type the following code

- Edit the dst IP address to change it to the Ubuntu’s/Fedora’s interface IP address.

- Save the file as “TCP_SYN_with_data.py” and close the file

- Run it by typing (check permission):

./ TCP_SYN_with_data.py

Go back to the Ubuntu/Fedora Linux VM

- Analyze Wireshark logs

- Make sure you can see the HTTP payload sent with TCP SYN packet

- Study the response from the Ubuntu/Fedora VM, what TCP flags are set on the response packet?

- How did the Kali VM respond?

## Deliverables

# Submit a report showing the following. Upload the report on Lab 1 assignment folder under Assignments on Canvas (one report per group).

# Snapshot of the Wireshark capture showing TCP SYN with data packets and response packets

- One paragraph explaining the response of the target machine to TCP SYN packets with data